Lab 5 Report

Name: Dhruv Sandesara

UT EID: djs3967

Section: 15295

Checklist:

Part 1 –

- Design file (.v) for the Ripple Carry Adder

- `timescale 1ns / 1ps

- //////////////////////////////////////////////////////////////////////////////////

- // Company:

- // Engineer:

- //

- // Create Date: 04/06/2018 02:57:29 PM

- // Design Name:

- // Module Name: RCA_4bits

- // Project Name:

- // Target Devices:

- // Tool Versions:

- // Description:

- //

- // Dependencies:

- //

- // Revision:

- // Revision 0.01 - File Created

- // Additional Comments:

- //

- //////////////////////////////////////////////////////////////////////////////////

- module RCA_4bits(

- input clk,

- input enable,

- input [3:0] A,

- input [3:0] B,

- input Cin,

- output [4:0]Q

- );

- wire cout0, cout1, cout2;

- wire [4:0] Data;

- full_adder f0 (.A(A[0]), .B(B[0]), .Cin(Cin),   .S(Data[0]), .Cout(cout0));

- full_adder f1 (.A(A[1]), .B(B[1]), .Cin(cout0), .S(Data[1]), .Cout(cout1));

- full_adder f2 (.A(A[2]), .B(B[2]), .Cin(cout1), .S(Data[2]), .Cout(cout2));

- full_adder f3 (.A(A[3]), .B(B[3]), .Cin(cout2), .S(Data[3]), .Cout(Data[4]));

- register_logic r0 (.clk(clk), .enable(enable), .Data(Data), .Q(Q));

- Endmodule

- `timescale 1ns / 1ps

- //////////////////////////////////////////////////////////////////////////////////

- // Company:

- // Engineer:

- //

- // Create Date: 04/06/2018 02:59:43 PM

- // Design Name:

- // Module Name: full_adder

- // Project Name:

- // Target Devices:

- // Tool Versions:

- // Description:

- //

- // Dependencies:

- //

- // Revision:

- // Revision 0.01 - File Created

- // Additional Comments:

- //

- //////////////////////////////////////////////////////////////////////////////////

- module full_adder(

- input A,B,Cin,

- output reg S,Cout

- );

- reg [1:0]out;

- always@(*)begin

- out = A+B+Cin;

- assign S= out[0];

- assign Cout= out[1];

- //{Cout,S}= A+B+Cin;

- end

- Endmodule

- `timescale 1ns / 1ps

- //////////////////////////////////////////////////////////////////////////////////

- // Company:

- // Engineer:

- //

- // Create Date: 04/06/2018 03:01:08 PM

- // Design Name:

- // Module Name: register_logic

- // Project Name:

- // Target Devices:

- // Tool Versions:

- // Description:

- //

- // Dependencies:

- //

- // Revision:

- // Revision 0.01 - File Created

- // Additional Comments:

- //

- //////////////////////////////////////////////////////////////////////////////////

- module register_logic(

- input clk,

- input enable,

- input [4:0] Data,

- output reg [4:0] Q

- );

- initial begin

- Q <= 0;

- end

- always @(posedge clk)

- if(enable)

- Q <= Data;

- Endmodule

- Test-bench

- `timescale 1ns / 1ps

- //////////////////////////////////////////////////////////////////////////////////

- // Company:

- // Engineer:

- //

- // Create Date: 04/06/2018 03:24:05 PM

- // Design Name:

- // Module Name: tb_RCA_4bits

- // Project Name:

- // Target Devices:

- // Tool Versions:

- // Description:

- //

- // Dependencies:

- //

- // Revision:

- // Revision 0.01 - File Created

- // Additional Comments:

- //

- //////////////////////////////////////////////////////////////////////////////////

- module tb_RCA_4bits;

- reg clk;

- reg enable;

- reg [3:0] A;

- reg [3:0] B;

- reg Cin;

- wire [4:0] Q;

- RCA_4bits uut (

- .clk(clk),

- .enable(enable),

- .A(A),

- .B(B),

- .Cin(Cin),

- .Q(Q)

- );

- initial begin

- clk = 0;

- enable = 0;

- A = 4'b0000;

- B = 4'b0000;

- Cin = 0;

- #50;

- enable = 1;

- A = 4'b0001;

- B = 4'b0101;

- Cin = 0;

- #50;

- A = 4'b0111;

- B = 4'b0111;

- Cin = 0;

- #50;

- A = 4'b1000;

- B = 4'b0111;

- Cin = 1;

- #50;

- A = 4'b1100;

- B = 4'b0010;

- Cin = 0;

- #50;

- A = 4'b1000;

- B = 4'b1000;

- Cin = 1;

- #50;

- A = 4'b1001;

- B = 4'b1010;

- Cin = 1;

- #50;

- A = 4'b1111;

- B = 4'b1111;

- Cin = 0;

- end

- always

- #5 clk = ~clk;

- endmodule

- Complete Table 1 from the simulation

| A[3:0] | B[3:0] | Cin | Sum[3:0] | Cout |
| --- | --- | --- | --- | --- |
| 0001 | 0101 | 0 | 6 | 0 |
| 0111 | 0111 | 0 | E | 0 |
| 1000 | 0111 | 1 | 0 | 1 |
| 1100 | 0100 | 0 | E | 0 |
| 1000 | 1000 | 1 | 1 | 1 |
| 1001 | 1010 | 1 | 4 | 1 |
| 1111 | 1111 | 0 | E | 1 |

Table 1.  Testcases for Ripple Carry Adder Verification

- Constraints File (Just the uncommented portion)

- ## Clock signal

- set_property PACKAGE_PIN W5 [get_ports clk]

- set_property IOSTANDARD LVCMOS33 [get_ports clk]

- create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk]

- ## Switches

- set_property PACKAGE_PIN V17 [get_ports {A[0]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[0]}]

- set_property PACKAGE_PIN V16 [get_ports {A[1]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[1]}]

- set_property PACKAGE_PIN W16 [get_ports {A[2]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[2]}]

- set_property PACKAGE_PIN W17 [get_ports {A[3]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[3]}]

- set_property PACKAGE_PIN W15 [get_ports {B[0]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[0]}]

- set_property PACKAGE_PIN V15 [get_ports {B[1]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[1]}]

- set_property PACKAGE_PIN W14 [get_ports {B[2]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[2]}]

- set_property PACKAGE_PIN W13 [get_ports {B[3]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[3]}]

- set_property PACKAGE_PIN V2 [get_ports {Cin}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Cin}]

- set_property PACKAGE_PIN U16 [get_ports {Q[0]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[0]}]

- set_property PACKAGE_PIN E19 [get_ports {Q[1]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[1]}]

- set_property PACKAGE_PIN U19 [get_ports {Q[2]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[2]}]

- set_property PACKAGE_PIN V19 [get_ports {Q[3]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[3]}]

- set_property PACKAGE_PIN W18 [get_ports {Q[4]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[4]}]

- set_property PACKAGE_PIN U18 [get_ports enable]

- set_property IOSTANDARD LVCMOS33 [get_ports enable]

- Simulation waveform for the above test-cases

Part 2 –

- All the equations for Ci’s and Si’s

- assign P=A^B;

- assign G=A&B;

- assign C[0]= Cin;

- assign C[1]= G[0]| P[0]&Cin;

- assign C[2]= G[1]| P[1]&G[0]| P[1]&P[0]&Cin   ;

- assign C[3]= G[2]| P[2]&G[1]| P[2]&P[1]&G[0]| P[2]&P[1]&P[0]&Cin ;

- assign C[4]= G[3]| P[3]&G[2]| P[3]&P[2]&G[1]| P[3]&P[2]&P[1]&G[0]| P[3]&P[2]&P[1]&P[0]&Cin;

- assign Data[0]= P[0]^C[0];

- assign Data[1]= P[1]^C[1];

- assign Data[2]= P[2]^C[2];

- assign Data[3]= P[3]^C[3];

- assign Data[4]= C[4];

- Design files (.v) for the Carry Lookahead Adder and Register Logic

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 04/06/2018 03:51:09 PM

// Design Name:

// Module Name: CLA_4bits

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module CLA_4bits(

input clk,

input enable,

input [3:0] A,

input [3:0] B,

input Cin,

output [4:0]Q

);

wire [4:0] Data;

wire [3:0] P;

wire [3:0] G;

wire [4:0] C;

assign P=A^B;

assign G=A&B;

assign C[0]= Cin;

assign C[1]= G[0]| P[0]&Cin;

assign C[2]= G[1]| P[1]&G[0]| P[1]&P[0]&Cin   ;

assign C[3]= G[2]| P[2]&G[1]| P[2]&P[1]&G[0]| P[2]&P[1]&P[0]&Cin ;

assign C[4]= G[3]| P[3]&G[2]| P[3]&P[2]&G[1]| P[3]&P[2]&P[1]&G[0]| P[3]&P[2]&P[1]&P[0]&Cin;

assign Data[0]= P[0]^C[0];

assign Data[1]= P[1]^C[1];

assign Data[2]= P[2]^C[2];

assign Data[3]= P[3]^C[3];

assign Data[4]= C[4];

register_logic r0 (.clk(clk), .enable(enable), .Data(Data), .Q(Q));

endmodule

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 04/06/2018 03:52:58 PM

// Design Name:

// Module Name: register_logic

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module register_logic(

input clk,

input enable,

input [4:0] Data,

output reg [4:0] Q

);

initial begin

Q <= 0;

end

always @(posedge clk)

if(enable)

Q <= Data;

Endmodule

- Test-bench

- `timescale 1ns / 1ps

- //////////////////////////////////////////////////////////////////////////////////

- // Company:

- // Engineer:

- //

- // Create Date: 04/06/2018 03:55:54 PM

- // Design Name:

- // Module Name: tb_CLA_4bits

- // Project Name:

- // Target Devices:

- // Tool Versions:

- // Description:

- //

- // Dependencies:

- //

- // Revision:

- // Revision 0.01 - File Created

- // Additional Comments:

- //

- //////////////////////////////////////////////////////////////////////////////////

- module tb_CLA_4bits();

- reg clk;

- reg enable;

- reg [3:0] A;

- reg [3:0] B;

- reg Cin;

- wire [4:0] Q;

- CLA_4bits uut (

- .clk(clk),

- .enable(enable),

- .A(A),

- .B(B),

- .Cin(Cin),

- .Q(Q)

- );

- initial begin

- clk = 0;

- enable = 0;

- A = 4'b0000;

- B = 4'b0000;

- Cin = 0;

- #50;

- enable = 1;

- A = 4'b0000;

- B = 4'b0101;

- Cin = 0;

- #50;

- A = 4'b0101;

- B = 4'b0111;

- Cin = 0;

- #50;

- A = 4'b1000;

- B = 4'b0111;

- Cin = 1;

- #50;

- A = 4'b1001;

- B = 4'b0100;

- Cin = 0;

- #50;

- A = 4'b1000;

- B = 4'b1000;

- Cin = 1;

- #50;

- A = 4'b1101;

- B = 4'b1010;

- Cin = 1;

- #50;

- A = 4'b1110;

- B = 4'b1111;

- Cin = 0;

- end

- always

- #5 clk = ~clk;

- Endmodule

- Complete Table 2 from the simulation

| A[3:0] | B[3:0] | Cin | Sum[3:0] | Cout |
| --- | --- | --- | --- | --- |
| 0000 | 0101 | 0 | 5 | 0 |
| 0101 | 0111 | 0 | C | 0 |
| 1000 | 0111 | 1 | 0 | 1 |
| 1001 | 0100 | 0 | D | 0 |
| 1000 | 1000 | 1 | 1 | 1 |
| 1101 | 1010 | 1 | 8 | 1 |
| 1110 | 1111 | 0 | D | 1 |

Table 2.  Testcases for Carry Lookahead Adder Verification

- Constraints File (Just the uncommented portion)

- ## Clock signal

- set_property PACKAGE_PIN W5 [get_ports clk]

- set_property IOSTANDARD LVCMOS33 [get_ports clk]

- create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk]

- ## Switches

- set_property PACKAGE_PIN V17 [get_ports {A[0]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[0]}]

- set_property PACKAGE_PIN V16 [get_ports {A[1]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[1]}]

- set_property PACKAGE_PIN W16 [get_ports {A[2]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[2]}]

- set_property PACKAGE_PIN W17 [get_ports {A[3]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {A[3]}]

- set_property PACKAGE_PIN W15 [get_ports {B[0]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[0]}]

- set_property PACKAGE_PIN V15 [get_ports {B[1]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[1]}]

- set_property PACKAGE_PIN W14 [get_ports {B[2]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[2]}]

- set_property PACKAGE_PIN W13 [get_ports {B[3]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {B[3]}]

- set_property PACKAGE_PIN V2 [get_ports {Cin}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Cin}]

- set_property PACKAGE_PIN U16 [get_ports {Q[0]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[0]}]

- set_property PACKAGE_PIN E19 [get_ports {Q[1]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[1]}]

- set_property PACKAGE_PIN U19 [get_ports {Q[2]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[2]}]

- set_property PACKAGE_PIN V19 [get_ports {Q[3]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[3]}]

- set_property PACKAGE_PIN W18 [get_ports {Q[4]}]

- set_property IOSTANDARD LVCMOS33 [get_ports {Q[4]}]

- set_property PACKAGE_PIN U18 [get_ports enable]

- set_property IOSTANDARD LVCMOS33 [get_ports enable]

- Simulation waveform for the above test-cases

Part 3 –

- Screenshots of the gate-level schematics for both the adder techniques

- Delay and area for both the adder techniques showing all the work

- Gate Delay (ns) Area

- XOR	 3 		6

- AND	3		4

- OR	2		4

- Table 3. Sample Delay and Area Values for Various

Part 2 :

18 And Gates

5 XOR Gates

10 OR Gates

Area is 18*4 + 5*6 + 10*4=  142

Max Path is 6 And&XOR gates deep which is 6*3 is 18ns

Part 1:

1 Full Adder has : 2* XOR, 2* And, and 1 OR;

MaX Delay per Adder is 1 XOR, 1AND, And 1 OR.

There fore

AREa is 2*6+2*4+4= 24

Delay is 3+3+2= 8

As we use 4 adders we get

Area is 4*24=96

Delay is 4*8= 32ns;

- Brief conclusion regarding the pros and cons of each of the techniques

Carry Ahead Looker: Pros is that it is fast due to minimal gate delay. Cons is that there are way too many gates used

Carry Ripple Ahead: Pros is that it uses less gates. Cons is that there is a O(n) gate delay as the MSB has to wait on the Carry bits from the lesser significat bits.

Note –> The Verilog codes and the uncommented portions of the constraint files should be copied in your lab report and the actual Verilog (.v), Constraint (.xdc) files and Bitstream (.bit) files need to be zipped and submitted as well on Canvas. You are not allowed to change your codes after final submission as the TAs may download the submitted codes or bitstream files from Canvas during checkouts. For the truth Table, K-maps minimizations and algebraic expressions, you are free to draw them on paper and then put the pictures in your lab report, but please make sure it is legible for the TAs to grade it properly.